# Literature Review Project Plan

# for Master in AI and Automation Programs at University West

# Title

Why do school buildings overheat? A literature review on thermal modelling and AI-assisted heating control in district-heated municipal buildings

# Student (Group 10)

Name: Hoang Vinh Nguyen E-mail: hoang-vinh.nguyen@student.hv.se Phone: 0722394531 Start year: 2025 Name: Mary Wagura E-mail: mary.wagura@student.hv.se Phone: 0769577809 Start year: 2025 Name: Kelvin Njoroge Wanjiku E-mail: kelvin-njoroge.wanjiku@student.hv.se Phone: 0767585622 Start year: 2025

# Course

PAI700 - Project Work in AI and Automation

# Amendment record

| Rev | Date | Purpose | Nature of change |
| --- | --- | --- | --- |
| 1 | 2026-10-11 | To supervisor | Initial version |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |

# Background

Municipalities own a lot of older buildings, and heating them is a big part of the energy bill. In Sweden, schools and preschools use about 5 TWh per year, and in 2006 the average school used about 213 kWh/m², of which 152 kWh/m² was heating [1]. Most of these buildings are heated by district heating through water radiators. A survey of 109 radiator systems in Gothenburg found average supply and return temperatures of 64 °C and 42 °C at the design outdoor temperature (-16 °C), and supply temperatures of 55 °C or lower whenever it was 5 °C or warmer outside[2] . So, the radiators are often not the limit; how the heat is controlled is. This review is tied to a feasibility study (PAI700) for Trollhättan Stad / Kraftstaden about AI-assisted heating optimization. The client describes a familiar pattern: fixed heating schedules, no outdoor temperature compensation, no account of internal gains, and no central overview of the buildings. The heating runs hard in the morning; the room overshoots, and around midday the ventilation ramps up to blow the extra heat out (“system fighting”). Because of thermal inertia, the radiators keep giving heat after the room is already warm enough. Older school studies describe the same symptoms. Measurements in three Växjö schools showed high indoor temperatures and high CO2 levels [3] . More recently, a study of seven low-energy schools found that measured energy use differed from the calculated use by between 44% below and 28% above, and that user-related parameters (indoor temperature, ventilation rate, ventilation operating time) mattered more than the technical ones [4] . Classroom models have also been used to compare ventilation strategies [5]. On the network side, recent work looks at how heat storage changes which heat sources are run in a district heating system [6]. Machine learning has also been tested for forecasting heat load on Swedish data: in Västerås, a support vector regressor gave the best hourly forecasts among six models (about 4% MAPE on the test data), and about 3–10% MAPE when it was retrained adaptively on the latest two or four weeks of data [7]. That work is at substation/network level, not for a single building, so it is a useful example of the method, but we need to be careful about how far it transfers to a classroom. Both [6] and [7] are the other end of the same pipe, mostly outside the scope of this review. A recent review of AI-based HVAC optimization points out that most studies so far are done in simulations, laboratories or small pilots, with few real-world deployments, and that practical barriers include missing sensors, legacy building management systems and models that do not transfer between buildings [8]. This matches our situation: we are working with simulation and cheap sensors, and no real building data yet.

# Purpose

For the municipality, the point is to decide before spending money whether AI-assisted heating is worth it. Operations optimization in school buildings is reported to give savings of roughly 10–30% [1], so it is worth a closer look. For the project, a literature review gives a solid base for the digital twin we are building which model types exist, what data they need, which control ideas have been tried, and where our assumptions are weak. It also helps to separate what is well documented from what is just “common knowledge” in our background notes.

# Aim(s)

Main goals: 1. Understand the problem. Summaries what the literature says about why school buildings with water-radiator heating overheat and waste energy (thermal inertia, control lag, internal gains, ventilation interaction, user behavior), and how big the effect is in Swedish/Nordic schools. - Collect benchmark numbers (energy use, temperatures, comfort) that can be used to validate the twin. 2. Compare models and control approaches. Give an overview of how a classroom/zone is modeled thermally and which control strategies are used or proposed (weather compensation, optimal start/stop, TRV/PID, forecast- and AI-based control). - Note what data and parameters each approach needs. - Note what savings or improvements have been reported, and how they were measured. 3. Draw lessons for the PAI700 twin. Point out what is useful for our simulation and POC (parameters, validation, low-cost sensors, old buildings) and list the gaps we found.

# Theory/related research

Theme; Found so far. Energy use in Swedish schools; Persson et al. [1] (the 2006 benchmark, operations optimization 10–30%); Simanic [5] (measured vs. calculated, user parameters). Radiator systems and district heating temperatures; Jangsten et al. [2]. Overheating, ventilation and indoor climate in classrooms; Larsson and Olsson [3]; Wheeler and Abend [5]. Thermal models of a room/building (e.g. RC / grey-box); Prince et al. [8] (review; mentions white-, grey- and black-box building models as the base for model predictive control, but is not a modelling paper itself). Control: optimal start/stop, MPC, ML forecasting, smart TRVs; Renuke et al. [7] (ML heat load forecasting, network level only); Prince et al. [8] (review of AI-based HVAC control: ML, deep learning, reinforcement learning, MPC). Occupancy and weather forecasting for heating; [7] uses outdoor temperature, hour of day and weekday as inputs, but no occupancy.

# Investigative question(s)

1. What causes heat overshoot and wasted heating energy in district-heated school buildings, according to the literature, and how large is it? 2. Which types of thermal models are used for a classroom/zone, and what inputs and parameters do they need? 3. Which control strategies (weather compensation, optimal start/stop, TRV/PID, forecast- or AI-based control) have been reported to reduce overshoot or energy use, and with what results? 4. What practical limits (old buildings, cheap sensors, missing data, validation) are reported for retrofitting such solutions? 5. Which of these findings can be used in the PAI700 digital twin, and what is still unknown?

# Limitations

- Focus on space heating in school/classroom-type municipal buildings with district heating and water radiators, in Sweden or similar Nordic climates. - Production-side district heating optimization (plants, storage, dispatch, pricing) is not covered, only mentioned for context or as an example of a forecasting method (e.g. [7]). - Cooling and domestic hot water are not covered. - Newer papers are preferred for methods (roughly the last 10–15 years); older ones are fine for classic references or Swedish school data. Sources in English or Swedish. - Cost and business analysis is left to the feasibility study.

# Schedule and activity plan

Week; PAI700 project work; Literature review; Load. 39; Meeting with the company (Sept 24); problem description and requirements; Not planned; read whatever looked relevant; Part-time. 40; Requirements and functions; Not planned; read whatever looked relevant; Part-time. 41; Functions, concepts and first prototype work; Collect the papers found so far; write and send this plan; Part-time. 42; Prototype; test and assessment; Read the papers on thermal models and control; notes feed the prototype; start the comparison table; Full-time. 43; Iterate prototype; test and assessment; Finish reading and the comparison table; write the first full draft; Full-time. 44; Final report and documents; submit; Revise the review, check references, hand in with the report; Part-time. 45; Buffer, only if needed; Buffer; -.

# References

[1]	A. Persson, M. Bratt, and S. Ekelin, Vägen till energieffektiva skolor. Sveriges Kommuner och Landsting, 2011.

[2]	M. Jangsten, J. Kensby, J.-O. Dalenbäck, and A. Trüschel, “Survey of radiator temperatures in buildings supplied by district heating,” Energy, vol. 137, pp. 292–301, 2017, doi: 10.1016/j.energy.2017.07.017.

[3]	R. Larsson and S. Olsson, “Improved indoor environment and ventilation in schools. A case study in Växjö, Sweden,” in Proceedings of the 13th AIVC Conference: Ventilation for Energy Efficiency and Optimum Indoor Air Quality, Nice, France, Sep. 1992.

[4]	B. Simanic, “Energy use in low-energy schools: Measurements and analysis of energy use and user-related parameters,” Lund, Sweden, Nov. 2020.

[5]	A. E. Wheeler and A. C. Abend, “Meeting the objectives of energy conservation and ventilation for acceptable indoor air quality in the classroom,” in Proceedings of IAQ 91 Healthy Buildings, Washington, DC, USA, Sep. 1991, pp. 215–220.

[6]	N. Scholliers, M. Ohagen, L. Schebek, I. Sass, and V. Zeller, “Modeling merit-order shifts in district heating networks: A life cycle assessment method for high-temperature aquifer thermal energy storage integration,” Energies (Basel)., vol. 19, no. 1, p. 212, 2025, doi: 10.3390/en19010212.

[7]	A. Renuke, S. Vouros, and K. Kyprianidis, “Machine learning assisted adaptive heat load consumption forecasting in district heating network,” in Proceedings of the 64th International Conference of the Scandinavian Simulation Society (SIMS 2023), in Linköping Electronic Conference Proceedings. Västerås, Sweden, Sep. 2023, pp. 393–404. doi: 10.3384/ecp200050.

[8]	Prince, B. Yoon, and P. Kumar, “Artificial intelligence-based optimization of HVAC systems: Advances in thermal comfort, energy efficiency, energy consumption, challenges, and research gaps,” Energy, vol. 364, no. 142240, Aug. 2026, doi: 10.1016/j.energy.2026.142240.